Assembly of 3D-Printed Parts

- Insert an M3 not in the circled slot on the gear and fasten an M3 bolt gently into it so that it won’t move.

- Insert the gear over the shaft of the motor. This has to be a tight fit. You may need to use a heat gun, blow drier, or a press.

- Use m4 nuts and bolts for the stepper motor and the motor cage.

- Use m2 nuts and bolts to secure the motor cage to the bridge.

- Insert an M3 nut in to the rack

- Place the rack into the rack base and insert an m3 bolt into the slot.

- Use M3 nuts and bolts to secure the rack base to the plate.

- Insert the rack through the motor cage

- Use M3 nuts and bolts to secure the bridge to the vent Box on the side facing the rack teeth.

  - Use M2 nuts and bolts to secure the two limit switches to the bottom of the bridge

- Use M3 nuts and bolts to secure the bridge and vent Box to the electronics box on the side opposite to the rack teeth.

- Insert the Lid over the electronics box.

- Secure the Arduino (red), Arduino Sound Module (white) and the optional Arduino Buck (green) Converter on the specified Blocks.

- Insert the above items as well as the Power Switch, RR and TV Adjustment Knobs, Stepper Motor Driver Universal Power Supply, and LED indicators in the electronics box.