- Part List:

  - NEMA 23 Stepper Motor: $39.99

https://www.amazon.com/gp/product/B00PNEPW4C/ref=ppx_yo_dt_b_asin_title_o04_s00?ie=UTF8&psc=1

b.

  - TB6600 Stepper Motor Driver: $10.75

https://www.amazon.com/gp/product/B0839BBFVZ/ref=ppx_yo_dt_b_asin_title_o07_s00?ie=UTF8&psc=1

  - ARDUINO UNO R3: $19.89

https://www.amazon.com/gp/product/B008GRTSV6/ref=ppx_yo_dt_b_asin_title_o04_s00?ie=UTF8&psc=1

  - Power Supply for Arduino: $8.69

https://www.amazon.com/gp/product/B01GD4ZQRS/ref=ppx_yo_dt_b_search_asin_title?ie=UTF8&psc=1

  - Power Supply for the Motor: $24.99

https://www.amazon.com/gp/product/B06XK2ZNKC/ref=ppx_yo_dt_b_asin_title_o05_s00?ie=UTF8&psc=1

  - Screws: $24.99

https://www.amazon.com/gp/product/B07H4MG7TC/ref=ppx_yo_dt_b_asin_title_o04_s00?ie=UTF8&psc=1

  - Jumper Cables: $6.90

https://www.amazon.com/gp/product/B01EV70C78/ref=ppx_yo_dt_b_search_asin_title?ie=UTF8&psc=1

  - Potentiometer (Optional):  $0.5

https://www.ebay.com/itm/20pcs-10K-ohm-Linear-Taper-Rotary-Potentiometer-Panel-pot-B10K-15mm/392123626642?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

- Connections:

| From Device | From | To | To Device |
| --- | --- | --- | --- |
| 15A Power Supply for Motor | L | L (Live) of Line to the wall | 120V Outlet |
|  | N | N (Neutral) of Line to the wall |  |
|  | Ground | Ground of Line to the wall |  |
|  | V- | GND on Stepper Driver | TB 6600 Stepper Driver |
|  | V+ | VCC on Stepper Driver |  |
| TB 6600 Stepper Driver | VCC | V+ | 15A Power Supply for Motor |
|  | GND | V- |  |
|  | A+ | Black Wire | Stepper Motor |
|  | A- | Green Wire |  |
|  | B+ | Red Wire |  |
|  | B- | Blue Wire |  |
|  | PUL+ | 5 | Arduino |
|  | PUL- | GND (Combine PUL-, DIR- and ENA- all and connect to GND of Arduino) |  |
|  | DIR+ | 2 |  |
|  | DIR- | GND (Combine PUL-, DIR- and ENA- all and connect to GND of Arduino) |  |
|  | ENA+ | 8 |  |
|  | ENA- | GND (Combine PUL-, DIR- and ENA- all and connect to GND of Arduino) |  |
| Stepper Motor | Black Wire | A+ | TB 6600 Stepper Driver |
|  | Green Wire | A- |  |
|  | Red Wire | B+ |  |
|  | Blue Wire | B- |  |
| Arduino | 2 | DIR+ | TB 6600 Stepper Driver |
|  | 5 | PUL+ |  |
|  | 8 | ENA+ |  |
|  | A1 | Middle Pin | Potentiometer |
|  | Vin | + |  |
|  | GND (Next to Vin) | - |  |
|  | GND (Next to 5V) | Combine PUL-, DIR- and ENA- all and connect to GND of Arduino | TB 6600 Stepper Driver |
|  | Power Supply Connection | 12V Power Adaptor |  |

- Settings:

  - TB 6600 Driver:

Set on the side controller of the driver

| 1 | On (Down) |
| --- | --- |
| 2 | Off (Up) |
| 3 | Off (Up) |
| 4 | Off (Up) |
| 5 | On (Down) |
| 6 | Off (Up) |

  - Arduino:

Code can be found at:

https://github.com/zoorage/Ventilator-Prototype.git

- Video of Earlier Prototypes:

https://www.youtube.com/watch?v=NUu_Q0IXxf4

https://www.youtube.com/watch?v=SI2yv-K0nfQ