Contributors to this project:

-Borzoo Farhang, DO, MS, MSHS

Assistant Professor of Anesthesiology

University of Vermont Medical Center

borzoo.farhang@gmail.com

-Alec Gill

-Tom Caulton

- Part List:

  - NEMA 23 Stepper Motor: $39.99

https://www.amazon.com/gp/product/B00PNEPW4C/ref=ppx_yo_dt_b_asin_title_o04_s00?ie=UTF8&psc=1

b.

  - TB6600 Stepper Motor Driver: $10.75

https://www.amazon.com/gp/product/B0839BBFVZ/ref=ppx_yo_dt_b_asin_title_o07_s00?ie=UTF8&psc=1

Or: 9.99

https://www.ebay.com/itm/Stepper-Motor-Driver-TB6600-4A-DC9-40V-for-Nema17-Nema23-Nema34-42-57-86mm/123554326786?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2648

  - ARDUINO UNO R3: $19.89

https://www.amazon.com/gp/product/B008GRTSV6/ref=ppx_yo_dt_b_asin_title_o04_s00?ie=UTF8&psc=1

  - Power Supply for Arduino: $8.69

https://www.amazon.com/gp/product/B01GD4ZQRS/ref=ppx_yo_dt_b_search_asin_title?ie=UTF8&psc=1

  - Power Supply for the Motor: $24.99

https://www.amazon.com/gp/product/B06XK2ZNKC/ref=ppx_yo_dt_b_asin_title_o05_s00?ie=UTF8&psc=1

  - Screws: $24.99

https://www.amazon.com/gp/product/B07H4MG7TC/ref=ppx_yo_dt_b_asin_title_o04_s00?ie=UTF8&psc=1

  - LED Indicator Red $14.91

https://www.digikey.com/product-detail/en/dialight/5571502203F/350-2411-ND/1832631

  - LED Indicator Green $16.49

https://www.digikey.com/product-detail/en/dialight/5571602203F/350-2414-ND/1832634

  - Limit Switches $0.79 (URBEST AC 250V 5A SPDT 1NO 1NC Momentary Hinge Roller Lever Micro Switches 3 Pins 10 Pcs)

https://www.amazon.com/gp/product/B00MFRMFS6/ref=ppx_yo_dt_b_asin_title_o07_s00?ie=UTF8&psc=1

  - Arduino Sound Module $1.56 (Oiyagai 5pcs DC 3.3-5V Passive Low Level Trigger Buzzer Alarm Sound Module for Arduino)

https://www.amazon.com/gp/product/B0777P6FN5/ref=ppx_yo_dt_b_asin_title_o06_s00?ie=UTF8&psc=1

  - Power Switch (still deciding)

  - Push Button $10.50

https://www.digikey.com/product-detail/en/e-switch/PV0H240SS-341/EG5391-ND/5964418

  - Jumper Cables: $6.90

https://www.amazon.com/gp/product/B01EV70C78/ref=ppx_yo_dt_b_search_asin_title?ie=UTF8&psc=1

  - Potentiometer (Optional):  $0.5

https://www.ebay.com/itm/20pcs-10K-ohm-Linear-Taper-Rotary-Potentiometer-Panel-pot-B10K-15mm/392123626642?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

  - Power Switch

  - Reset Button (Momentary Relay Module)

  - Arduino Sound Module

  - There are a few sets of wires to combine like the + of potentiometer 1 and 2. You can use any of the technics below:

    - a crimping tool ($40.39) (https://www.amazon.com/gp/product/B002AVVO7K/ref=ppx_yo_dt_b_search_asin_title?ie=UTF8&psc=1) with Dupont connectors (27.99) (https://www.amazon.com/gp/product/B078RRPRQZ/ref=ppx_yo_dt_b_search_asin_title?ie=UTF8&psc=1)

    - Solder cables together and use electrical tape around it

    - Use a breadboard ($11.99) (https://www.amazon.com/gp/product/B0727X6N9D/ref=ppx_yo_dt_b_search_asin_title?ie=UTF8&psc=1)

    - Use PCB board ($12.99) https://www.amazon.com/gp/product/B079DN31SW/ref=ppx_yo_dt_b_search_asin_title?ie=UTF8&psc=1

- Connections:

| From Device | From | To |  | To Device |
| --- | --- | --- | --- | --- |
| 15A Power Supply for Motor | L | L (Live) of Line to the wall |  | 120V Outlet |
|  | N | N (Neutral) of Line to the wall |  |  |
|  | Ground | Ground of Line to the wall |  |  |
|  | V- | GND on Stepper Driver |  | TB 6600 Stepper Driver |
|  | V+ | VCC on Stepper Driver |  |  |
| TB 6600 Stepper Driver | VCC | V+ |  | 15A Power Supply for Motor |
|  | GND | V- |  |  |
|  | A+ | Black Wire |  | Stepper Motor |
|  | A- | Green Wire |  |  |
|  | B+ | Red Wire |  |  |
|  | B- | Blue Wire |  |  |
|  | PUL+ | 5 |  | Arduino |
|  | PUL- | GND (Combine PUL-, DIR- and ENA- all and connect to GND of Arduino) |  |  |
|  | DIR+ | 2 |  |  |
|  | DIR- | GND (Combine PUL-, DIR- and ENA- all and connect to GND of Arduino) |  |  |
|  | ENA+ | 8 |  |  |
|  | ENA- | GND (Combine PUL-, DIR- and ENA- all and connect to GND of Arduino) |  |  |
| Stepper Motor | Black Wire | A+ |  | TB 6600 Stepper Driver |
|  | Green Wire | A- |  |  |
|  | Red Wire | B+ |  |  |
|  | Blue Wire | B- |  |  |
| Arduino | 2 | DIR+ |  | TB 6600 Stepper Driver |
|  | 5 | PUL+ |  |  |
|  | 8 | ENA+ |  |  |
|  | A0 | Middle Pin Potentiometer 1 |  | Potentiometer 1 and 2 |
|  | A1 | Middle Pin Potentiometer 2 |  |  |
|  | Vin | + (Combine + of potentiometer 1 and 2 and connect to Vin) |  |  |
|  | GND (Next to Vin) | - (Combine - of potentiometer 1 and 2 and connect to GND) |  |  |
|  | GND (Next to 5V) | Combine PUL-, DIR- and ENA- all and connect to GND of Arduino |  | TB 6600 Stepper Driver |
|  | Power Supply Connection | 12V Power Adaptor |  |  |
| Potentiometer 1 and 2 | Middle Pin Potentiometer 1 | A0 | Arduino |  |
|  | Middle Pin Potentiometer 2 | A1 |  |  |
|  | + (Combine + of potentiometer 1 and 2 and connect to Vin) | Vin |  |  |
|  | - (Combine - of potentiometer 1 and 2 and connect to GND) | GND (Next to Vin) |  |  |

- Settings:

  - TB 6600 Driver:

Set on the side controller of the driver

| 1 | Off (Up) |
| --- | --- |
| 2 | Off (Up) |
| 3 | Off (Up) |
| 4 | Off (Up) |
| 5 | Off (Up) |
| 6 | Off (Up) |

  - Arduino:

Code can be found at:

https://github.com/zoorage/Ventilator-Prototype.git

- Video of Earlier Prototypes:

https://www.youtube.com/watch?v=NUu_Q0IXxf4

https://www.youtube.com/watch?v=SI2yv-K0nfQ